CPE301 – SPRING 2020

# MIDTERM 1

Student Name: Minsung Cho

Student #: 2001446442

Student Email: chom3@unlv.nevada.edu

Primary Github address: https://github.com/cho-minsung

Directory: https://github.com/cho-minsung/midterm1

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

- LM35

- ATMEGA328PB

- Arduino shield

the LM35 isn’t connected to 32. It is just for the diagram’s sake.

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

All the previous codes were used as initial codes.

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

/*

* midterm1.c

*

* Created: 3/30/2020 7:11:20 PM

* Author : Minsung Cho

*/

#define F_CPU 16000000

#define BAUD 9600

#include <util/setbaud.h>

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/setbaud.h>

#include <stdio.h>

uint8_t ascii_2_int(uint8_t n)

{

if(n >= 48 && n <= 57) //outside of integers

{

send_byte();

return n - 48;

}

return 1;

}

void send_byte(uint8_t byte)

{

if(UCSR0B & (1<<TXEN0))

{

while(!(UCSR0A & (1<<UDRE0)));

UDR0 = byte;

}

}

void send_str(char string[])

{

int i = 0;

while (string[i] != '\0')

send_byte(string[i++]);

}

char get_byte()

{

while(!(UCSR0A & (1<<7)));

return UDR0;

}

uint16_t analog_conv()

{

ADCSRA |= (1<<6);				//ADSC is enabled for conversion.

while (!(ADCSRA&(1<<4)));

ADCSRA |= (1<<4);				//ADIF flag is cleared.

return ADC;

}

float dig_2_temp(uint16_t temp, char mode)

{

float temp_converted = temp*110/1023;

switch (mode) {

case 't':

return temp_converted;			//Temperature in Celsius is returned.

case 'T':

default:

return temp_converted*1.8 + 32;	//Temperature in Fahrenheit is returned.

}

}

int main(void)

{

//ADC is enabled here

DDRC &= ~(1<<4);		//pin4 is set as input.

ADMUX = 0xC4;			//1100 0100, internal 1.1V ref, right justified, ADC4.

ADCSRA = 0x00;			//0000 0000, free running.

ADCSRB = 0x87;			//1000 0111, enables ADC, prescaler of 128.

//UART is enabled here

DDRD |= 0x02;			//port is set.

UCSR0C = 0x06;			//Asynchronous, no parity.

//transmitter is enabled here

UCSR0B |= ~(1<<RXEN0);

//receiver is enabled here

DDRD &= ~1;

UCSR0B |= (1<<RXEN0);

//LED is enabled here

DDRB |= (1<<2);

PORTB &= ~(1<<2);

DDRB |= (1<<5);

PORTB &= ~(1<<5);

//timer0 and timer1 are initialized here

OCR1A = 15624;			//An approximation of 1 second for 16Mhz 1024 prescaler.

TIMSK1 = 0x02;			//compare match A interrupt is enabled.

TIFR1 |= 0x02;			//interrupt flag is cleared.

sei();

TCCR1B = 0x0D;			//timer1 is set as CTC mode with 1024 prescaler.

TCCR1A = 0x00;

TCNT1 = 0;

//various arrays are set

const char menu[] = "h: Help screen \n"

"t: Displays temperature in C \n"

"T: Displays temperature in F \n"

"o: Turns ON LED on PB5 \n"

"O: Turns OFF LED on PB5 \n"

"s: Sends a string to the terminal \n"

"i: Sends an integer to the terminal \n \n";

volatile char temp_string[5];

volatile char integer[1];

//menu will pop up

send_str(menu);

while(1)

{

char instruction = get_byte();

if(instruction == 'h')

send_str(menu);

else if(instruction == 't' || 'T')

{

uint16_t temperature = analog_conv();

float temp_converted = dig_2_temp(temperature, instruction);

snprintf(temp_string, 5, "%f\n", temp_converted);

send_str(temp_string);

send_byte('\n');

}

else if( instruction == 'o')

PORTB &= ~(1<<5);

else if(instruction == 'O')

PORTB |= (1<<5);

else if(instruction == 's')

send_str("I love Dr. Venki");

else if(instruction == 'i')

{

snprintf(integer, 1, "%d\n", 1);

send_str(integer);

send_byte('\n');

}

}

}

ISR(TIMER1_COMPA_vect)

{

TIFR1 |= 0x02;

int delay_time = 0;

delay_time = (delay_time + 1) %1;

if (delay_time == 0)

PORTB ^= (1<<2);

}

- SCHEMATICS

see the schematic above.

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=wFgkiGTEMdM

- GITHUB LINK OF THIS DA

https://github.com/cho-minsung/midterm1

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Minsung Cho