CPE301 – SPRING 2020

# MIDTERM 2

Student Name: Minsung Cho

Student #: 2001446442

Student Email: chom3@unlv.nevada.edu

Primary Github address: https://github.com/cho-minsung

Directory: https://github.com/cho-minsung/midterm2

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB		FIT0450 Micro DC Motor with Encoder-SJ01	Potentiometer

Wires			Breadboard					TB6612FNG

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

if (Flag == 1)

{

PORTD |= (1<<DDD5);

PORTD &= ~(1<<DDD4);

_delay_ms(5000);

Flag = 1;

}

else

{

PORTD &= ~(1<<DDD5);

PORTD |= (1<<DDD4);

_delay_ms(5000);

Flag = 1;

}

CW and CCW code

ISR(TIMER1_CAPT_vect)

{

revTick = ICR1;

TCNT1 = 0;

revCtr++;

T10vs2 = 0;

}

ISR(TIMER1_OVF_vect)

{

t10vs2++;

}

int main()

{

while (1)

{

update_ADC();

scale_set_speed();

Period = (uint32_t)(revTick) + ((uint32_t)T10vs2 * 0x10000L);

calculate_speed_rpm();

USART_SendString("RPM ");

snprintf(outs, sizeof(outs), "%f rpm ", (float)RPM);

USART_SendString(outs);

USART_SendString(" \r\n");

}

}

calculate 1x speed demo code

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

int main(void)

{

PORTC |=  (1<<3); //pull up

DDRD |= 0xF0; //PORTD as Output

//CCW: AIN1 = L, AIN2 = H, PWM = H

//CW: AIN1 = H, AIN2 = L, PWM = H

while(1)

{

PORTD = 0x10; //counter clock wise

_delay_ms(5000);

PORTD = 0x00; //no movement

_delay_ms(5000);

PORTD = 0x20; //clock wise

_delay_ms(5000);

PORTD = 0x30; //no movement

_delay_ms(5000);

}

}

clock wise and counter clock wise DC motor code

#define F_CPU 16000000UL

#define BAUD 9600

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <util/setbaud.h>

#include <stdio.h>

//given adc functions from previous assignments

void adc_init();

void update_ADC();

volatile float poten;

//given USART functions from previous assignments

void USART_init();

void USART_SendString(volatile char *data);

//given Timer0 & 3 functions from previous assignments and lectures

void Timer0_init();

void Timer3_init();

//RPM floating point data functions

float RPM_1x();

float RPM_2x();

float RPM_4x();

volatile float revTick1;

volatile float revTick4;

static char data[10];

int main(void)

{

//the port setup

DDRD |= (1<<6);	//the speed control

DDRD &= ~(1<<4);	//direction control

DDRD &= ~(1<<5);

PORTD &= ~(1<<4);

PORTD &= ~(1<<5);

DDRB |= (1<<0);	//encoder speed capture for 1x and 2x

PORTB &= ~(1<<0);

DDRE |= (1<<2);	//encoder speed capture for 4x

PORTE &= ~(1<<2);

DDRC |= (1<<0);	//the potentiometer

PORTC &= ~(1<<0);

//initiation

adc_init();

USART_init();

Timer0_init();

Timer3_init();

sei();

while (1)

{

if (poten < 5)

{

USART_SendString("1x encoding RPM: 0 \n");

//USART_SendString("2x encoding RPM: 0 \n"); enable these for 2x or 4x

//USART_SendString("4x encoding RPM: 0 \n");

}

else

{

USART_SendString("1x encoding RPM: ");

snprintf(data, sizeof(data), "%f\n\r", RPM_1x());

USART_SendString(data);

USART_SendString(" \n");

/*

USART_SendString("2x encoding RPM: ");

snprintf(data, sizeof(data), "%f\n\r", RPM_2x);

USART_SendString(data);

USART_SendString(" \n");

*/

/*

USART_SendString("4x encoding RPM: ");

snprintf(data, sizeof(data), "%f\n\r", RPM_4x);

USART_SendString(data);

USART_SendString(" \n");

*/

}

_delay_ms(1000);

}

}

void adc_init(void)

{

ADMUX =  (0<<REFS1) | (1<<REFS0) | (1<<ADLAR) | (0<<MUX2) | (0<<MUX1) | (0<<MUX0);

ADCSRA = (1<<ADEN) | (0<<ADSC) | (0<<ADATE) | (0<<ADIF) | (0<<ADIE) | (1<<ADPS2) | (0<<ADPS1) | (1<<ADPS0);

}

void update_ADC(void)

{

unsigned char i = 5;

poten = 0;

while(i--)

{

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

poten += ADC;

}

poten = poten/5;

}

void USART_init(void)

{

UBRR0H = UBRRH_VALUE;

UBRR0L = UBRRL_VALUE;

UCSR0C = _BV(UCSZ01)|_BV(UCSZ00);

UCSR0B = _BV(RXEN0)|_BV(TXEN0);

}

void USART_SendString(volatile char *data)

{

while((*data != '\0'))

{

while(!(UCSR0A & (1<<UDRE0)));

UDR0 = *data;

data++;

}

}

void Timer0_init(void)

{

TCCR0A |= (2 << COM1A0);

TCCR0A |= (0 << WGM10);

TCCR0B |= (0 << WGM12);

TIMSK0 |= (0x01);

TCNT0 = 0x00;

OCR0A = 0x00;

TCCR0B |= (0x02 << CS10);

}

void Timer3_init(void)

{

TCCR3A = (0);

TCCR3B = (1 << WGM32);

OCR3A  = (623);

}

float RPM_1x()

{

float tempRPM1 = 0.0;

for (int i = 0; i < 20; i++)

{

TCNT3 = 0x00; //see the note for RPM4x for more info

TCNT4 = 0x00;

TIMSK4 |= (1 << 5);

TIMSK3 |= (1 << 1);

TCCR4B |= (0x07) | (1 << 6);

TCCR3B |= (0x04 << 0);

TCCR3B &= ~(0x07);

TCCR4B &= ~(0x07);

tempRPM1 = tempRPM1 + revTick4 * 60 / 96;

}

return tempRPM1 / 20;

}

float RPM_2x()

{

float tempRPM2 = 0.0;

for (int i = 0; i < 20; i++)

{

EICRA = 0x01; //see the note for RPM4x for more info

EIMSK = 0x01;

TIMSK4 = (1 << 5);

TIMSK3 = (1 << 1);

TCNT3 = 0x00;

TCNT4 = 0x00;

TCCR4B = (0x07) | (1 << 6);

TCCR3B = (0x04 << 0);

EIMSK = 0x00;

EICRA = 0x00;

TCCR3B &= ~(0x07);

TCCR4B &= ~(0x07 | (1 << 6));

tempRPM2 = tempRPM2 + revTick4 * 60 / 192;

}

return tempRPM2 / 20;

}

float RPM_4x()

{

float tempRPM4 = 0.0;

for (int i = 0; i <20; i++)

{

EICRA = 0x05;

EIMSK = 0x03; //Int1 is enabled

TIMSK1 = (1 << 5); //The input capture interrupts are enabled

TIMSK4 = (1 << 5);

TIMSK3 = (1 << 1); //The output capture interrupt is enabled

TCNT3 = 0x00;	//Timer3 is reset

TCNT1 = 0x00;   //Counter1 & 4 are reset

TCNT4 = 0x00;

TCCR1B = (0x07) | (1 << 6); //The counter1 & 4 are set as rising edges

TCCR4B = (0x07) | (1 << 6);

TCCR3B = (0x04);			//The timer3 scaler is set

EIMSK = 0x00; //interrupts are reset

EICRA = 0x00;

TCCR3B &= ~(0x07);	//clocks are cleared

TCCR1B &= ~(0x07 | (1 << 6));

TCCR4B &= ~(0x07 | (1 << 6));

tempRPM4 = tempRPM4 + (revTick1 + revTick4) * 60 / 384;

}

return tempRPM4 / 20;

}

ISR(USART0_TX_vect)

{

//from the previous assignments

UCSR0B &= ~((1 << TXCIE0) | (1  << TXEN0));

}

ISR(ADC_vect)

{

ADCSRA &= ~(1 << ADIE); //ADC interrupt is disabled

OCR0A = poten;

}

ISR(TIMER0_OVF_vect)

{

ADCSRA |= (1 << ADIE); //ADC interrupt is enabled

}

ISR(TIMER1_CAPT_vect)

{

//detects the inB

TIMSK1 &= ~(1<<5);

revTick1 = (ICR1);

}

ISR(TIMER3_COMPA_vect)

{

PORTE |= (1 << 0); //Timer1 & 4 capture interrupt is set

PORTB |= (1 << 0);

TIMSK3 &= ~(1 << 1);

}

ISR(TIMER4_CAPT_vect)

{

//detects the inA

TIMSK4 &= ~(1<<5);

revTick4 = (ICR4);

}

2x and 4x are commented out.

- SCHEMATICS

- I used Timer0 PWM for controlling the speed of the motor to PD6

- I used pins PD5 and PD4 for direction control

- I used Timer1 ICP PB0 pin for encoder speed capture for 1x and 2x speed

- I used Timer3 ICP PE2 pin for encoder speed capture for 4x speed

- I used PC0 for the potentiometer

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=295bseZTShs

- GITHUB LINK OF THIS DA

https://github.com/cho-minsung/midterm2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Minsung Cho